Using the Franklin Compiler to create code for the 8044 using the

DCX on-chip multi-tasking operating system.

---------------------------------------------------------------------

This is a very brief discussion about how to use the Franklin C

compiler (my version is v.300e) to work with the 8044 multi-tasking

envoironment. I have used the Franklin compiler using large model

to produce tasks that run without conflict, but it was not easy. This

paper is not intended to spell out every detail (maybe later), and

it assumes that the programmer is very familar with the 8044, the

Franklin c-51 compiler, the assembler and the linker (I use Intel's

assembler and linker, more about that later).

There are many things to do, where to start? Well the easiest thing

to do is to make a copy of the large library (C51L.LIB), say to BBL.LIB.

Next delete the ?C_STARTUP module from BBL.LIB using the librarian. I

used the Intel Librarian, but I am sure Franklin's work just fine.

The next thing to do is to create a module that will contain the

C initialization routine, your task ID's and vectors to your C code.

Be sure to name the module you create as ?C_STARTUP using the assembler's

NAME directive.

Declare your first IDD in the absolute segment at 0FFF0H (the first

ITD/IDD chain entry. I used the first entry as the IDD, but it could

just as well have been an ITD. Let me make an important note right here

and now. All ITDs and IDDs must exist in absolute segments. This is

because the pointer to the Task code is relative pointer, and must be

calculated at assembly time.

Using Intels ITD macro, I used this:

CSEG      AT 0FFF0H

IDD:

%IDD(SYS_CLOCK_PRIORITY,SYS_CLOCK_PRIORITY,SYS_BUFFER_SIZE,SYS_RESERVED_MEM,ITD1)

Note: SYS_CLOCK_PRIORITY and other constants you need to create.

Notice the next IDD/ITD pointer, ITD1, well where is that? In another

absolute segment of course. In this example I am using the Bitbus board with

memory option B (for uploading and downloading code), and my first available

downloadable code instruction is at 8500h. So here is the first ITD.

CSEG       AT 8500H

ITD1:

%ITD(TASK1_INIT, 15, 081H, 110B, 2, 00000B, ITD2)

;------------------ PERFORM C INITIALIZATION, JUMP TO TASK1 CODE

TASK1_INIT:

LJMP    ?C_START

MAIN:           LJMP    TASK1

;------------------

Let me make an important mental note here, the highest priority

task is the one that initializes all variables. After all tasks

have been created, the highest priority task will run first, thus

the init code must be in that tasks control thread.

As you can see the ITDs relative offset to the tasks first code

address is TASK1_INIT. From there, ?C_START is called to initalize

any static constant ect. Then the trick! When ?C_START has comleted,

it jumps to MAIN, which is normally your main() function. But using

the above label MAIN, execution continues here, then the jump is

encountered and control passes to your void task1(void).

Notice that another ITD is to be process (ITD2). The following lines

come right after the above code, and thus are in the current abosulte segment.

ITD2:

%ITD(TASK2_INIT, 10, 082H, 101B, 1, 00000B, ITD2+1)

TASK2_INIT:

LJMP    TASK2

Now that is much better isn't it? Control passes to TASK2_INIT, then

to void task2(void). More ITDs can be added, but as you see, I have

terminated the ITD chain by setting the pointer to the next itd in the

usual manner (ITD2+1).

Notice in the ITDs that the register banks and priority level have been

selected carefully, and that these values will come into play when creating

the actual C code. The register banks specified are especially important

and major problem with occur if you mismatch the c code and ITD chain

register banks. Much more about that later. Also note the stack sizes

declared must be adequate for the C code to run correctly.

Now you need to declare some of the above public, and also declare

some of the stuff external (i.e ?C_START which in in the large library),

these details I leave to you.

Now let's discussion the 8044 internal memory requirements. First,

register bank 0 is off limits to other tasks which are not at priority

4 (equal to the RAC task). The C-51 compiler declares register BANK 0

automatically and will not permit overlapping there. I am not sure

why the do this, but it is not important as fas as the 8044 programmer

is concerned.

The other register banks are at this point available for storage. If you

use the #pragma directive, register banks will automatically be reserved

for use by the c code. If you do not use the register banks for code they

can be used for DATA storage locations. More about that later.

Now to discuss other areas of memory. You will need to create a module

which declares all reserved memory used with the use of absolute segment

declarations. Where to stay out of you ask? Well there are several areas.

a) DATA 20h - 29h

Note: Bit addressable area from 2ah to 2fh is available for use.

b) DATA 30h - 4bh

Note: with the use of an IDD, memory can be reserved from 4c updward

in DATA memory for I think 50 some bytes.

c) CODE  0 - fffh

d) XDATA 0 - 300h

Now that that memory is reserved, you may want to push all of the

relocatable code up from 1000h. Just reserve the amount of memory

you need at to push up the code. For instance, I need all code to

be placed at 8500h on up, so I reserved the following:

CSEG AT 1000H

DS 7500H

Notice that the ITD stuff earlier begins at 8500h, so all relocatable

code is push up beyond that stuff. There is no nice way to tell the linker

where to begin the address assignments for all relocateable segments, so

this is probably the cleanest way. Wouldn't it be nice if a directive

were created for the linker?

On to the task code:

#pragma LARGE

#pragma OPTIMIZE(3)

.

. include files

.

.

. Global variable declarations, function prototypes ect...

.

#pragma RB(2)

.

. TASK 1 EXCLUSIVLY CALLED FUNCTIONS!

.

void task1 (void) using 2

{

.

. statements

.

}

#pragma RB(1)

.

. TASK 2 EXCLUSIVLY CALLED FUNCTIONS!

.

void task2 (void) using 1

{

.

. statements

.

}

No for the long awaited register bank discussion. As you can see the

#pragma compiler directive is used to switch to register bank #2 useage

before compiling task1 functions. The reason is this, the compiler

generates register specific memory pushes within the code according

to the current register bank setting (default=0). So basically, if

you do not tell the compiler what register bank to use, you are looking

for trouble. The RAC task uses register bank 0, so stay out of there.

Anyhow, the complier see the #pragma directive an will at times generate

register specific pushes for all functions until either a 'using' directive

is encountered, or another #pragma RB(n) is encountered. This basically

means that your multiple tasks cannot share functions. There is one

exception to the rule, two tasks at the same priority can share the

same register bank, and thus they can share functions. If this is the

case, use the #pragma RB(n) to include both tasks, and all of their

associated functions.

As far as the example goes, you can see that register bank #2 is used

for task code. This was not a random selection. Remember the ITD stuff?

There I specified that task1 was a priority 2 task, and that it will

use register bank #2. The sample reasoning applys with task2.

If you want to call a library function from within two different tasks

which are at different priority levels, you will have problems. The

reason is two fold. The DCX operating system is premptive, and the other

is the arguement passing mechanism used by the Franklin C compiler. If

a low priority task is running, and loads the arguements to be passed

to a library function (which exist at fixed addresses), a higher priority

task may prempt the lower priority task and overwrite those arguements such

that when the lower priority task resumes executions, the library arguements

are meaningless.

This can be prevented using several techniques. The first is to do resource

sharing uses internal bits as semaphores. The other is to disable all

interrupts with using a library function. I say if the function is fairly

quick, disable interrupts, and for large time consuming functions use

semaphores. I leave that to you.

The disable interrupts trick will prevent any task switching from taking

place. Not even the RAC task waiting on an interrupt will execute. This

can present problems when executing time consuming functions and receiving

messages from the network. You must not hog the processor for more than

I would say 9 milliseconds. If you take 10 or more milliseconds to release

the processor, you run the risk of creating a protocol error. The master

node will try twice, and wait 10 millieseconds each time. If it fails on

the second try, the node is declared off line and a protocol error is

generated. So in general, if your on the edge, use semaphores.

Assembly interface functions can be written easily, and can also play

a role in the above mentioned function sharing problems. What you can

do as a nice trick is disable all interrupts, call the assemble language

routine (written flawlessly), and have it enable interrupts once it

has secured the arguements within its register (Acc,B,DPTR,R0-R7). This

is just one trick that can be played.

What would be best is probably this, all functions called (including

library functions) are supplied with a parameter block number. Thus

functions whos arguements exist in external memory can calculate an

offset to the appropriate arguement block, and proceed as normall.

For example the function strcat prototype would look like this:

extern char *strcat(char arg_bank, char *s1, char *s2, int n);

Calling the function from task1 would look like:

extern char *strcat(1,ts1,ts2,80);

And like magic, no more problems using any multitasker. Heck you

could even do recursion. Well unless someone from Franklin sees the

advantage, forget this possiblity, you will have to live with

semaphores and the disable interrupts trick.

Onward. Well what about the DCX system services? Well the answer to

your long awaited question is this, you must create an assembly language

interface for the DCX services. I have implemented several of these

services and they work nicely. I use the disable interrupts trick, and

re-enable the interrupts just before the DCX system call. It is relatively

easy to write the routines, and I plan on implementing all DCX services.

This may have already been done by someone, or by Franklin, but I prefer

to stumble through myself, thanks.

One thing to be aware of. When linking, you will have to either carefully

manage overlays or disable them altogether. Personally I like to disable

all overlaying of data objects. Who wants to analyze the overlays after

each new function/variable is added? One word of caution, using the

Franklin linker L51 v2.4 and the NOOVERLAY directive, I was unable to prevent

overlays. This may be a bug, who knows, just be careful (look at the

*.m51 file gererated by the linker. If you have intel's RL51, you

should not have a problem with disabling overlays.

Well that just about covers the topic. I have left out many topics relating

to the DCX multitasking system, and how to implement semaphores in it, but

I hope you now have an idea of the magnitude of the project at hand.

When I have completed the DCX system interface routines, for a fee, I

can supply you with routines and all related information. I have

not determined how much to charge at this point, but I am open to

your suggestions. FAX me if you are interested (see end of text).

-------------------------------------------------

Since I am now in the advertising business, I would now like to mention

a little product we have been working on for quite some time. I will not

go into all of the details on how it works, but let me throw out a few

things of interest to you Bitbus programmers/users out there.

Below I have listed several important problem areas which can occur

when using the 8044 and Bitbus. If you are using Bitbus, it is worth

the effort to follow the discussion. Please note that the following

statements do not apply to the 8052 or other chip sets implementing

the Bitbus protocol, they pertain only to the use of the 8044.

a) The master node must encorporate a majority of the logic of the

entire network control (machine tools, robots, ect...) along with

the normal required Bitbus network management logic.

Encorporating all of this logic is a nightmare at best when controlling

a complex system in 8044 code, and a bad dream when using an IBM pc as

the master (more about that in a second).

Here is an example. Lets say you are talking to several nodes, delivering

messages simultaneously, and a protocol error is returned, your

recommended slave node recovery sould be:

1) Resync the node.

2) Sometime later (assuming you have other things to talk to), attempt

to reset the node.

3) Since the master continues to poll a successful delivery of a

reset message, the master should resync the node again to

reset the number of outstanding messages, and to quit polling the

slave node (wasting network bandwidth).

4) So resync the node.

5) Sometine later (500 milliseconds min.), attempt to send messages to

to the slave again.

Not only the above must be accomplished, your code must also recover

from this disaster and possibly restart a message sequence which

was in progress. With all the above tasks at hand, you really need

a multi-tasker running on the PC. If you need better peformance,

you need a faster computer and a mutli-tasker, thus driving up

the entire system price.

Most IBM pc boards (PC344 for example) use a FIFO to pass messages

between the 8044 and the IBM pc. The mechanism quickly comes to a

halt if the master is out of buffers (PC344 being the master node).

Since you cannot reset the board with hardware, you must reset the

IBM and start from scatch (that is if someone is available to hit the

power switch). That is not the only problem with the PC344 board,

it is also slow as compared with the potential throughput of the

8044 on a 375k baud channel (due to the interrupt driven FIFO

routines).

So the PC344 is not the solution. Although a work around the

internal buffer problem, and the FIFO interrupt routines can

be accomplised (you can write a custom high speed polling task

on the PC344 to manage the FIFO, available for a fee...), the

PC344 is not the solution.

b) There is no obbious mechanism for sharing information between

slave processors. If you want the slaves to share information,

it is the responsibility of the master node or extension device

(PC344) to accomplish this. This goes back to the above mentioned

problems.

c) Unless you are doing buffer management on each node in the network,

the only way to prevent disasterous results on the network is to

send 1 message from the master, to a slave, wait for the reply,

then send another after you process the reply. This basically

means the bandwidth of the network is almost completely wasted.

Each slave should have at least 3 message buffers available for

processing (1 of which is allocated and not returned), thus

leaving 2 more message buffers for master slave communications).

Those extra buffers might as well be used for variables in the

above network operational description. If you are attempting a

fancy scheme were the master can deliver message from slave to

slave, the master must wait for each reply before generating another

order message, or to inject it's own order messages onto the network.

If you ignore this general rule, and attempt to flood the network

with messages, you will soon experience the "locked buffer" condition

in which the master node goes into an infinite loop attempting to

deliver a message to a slave node that does not have a free buffer,

or has crashed for some reason (and left the SIU in auto mode).

This only occurs if the master does not have a free buffer, but

if you are flooding the network, it will happen almost certainly

happen.

Well that is just a few of the major problems that can be encountered

using the 8044 and Bitbus, there are more. I have not compiled a list of

all the pitfalls but may consider doing so if requested.

Now the cure.

-------------

The cure for the above problems is easily accomplished using our

8044 software running on the master node. Our software deals with

call of the above mentioned issues. The network is configured

once, and can be installed in a PROM socket. Each time the master

nodes is re-booted, it establishes a connection with all nodes on

the network, and begins scheduling messages.

Full buffer management/utililization is realized. All available buffers

can be used (on the master and on all slaves) for network transactions

on demand. No more locked buffer conditon, thus no more permanent system

halts.

All other nodes on the network are slave nodes which can communicate

with each other at high speeds. The throughput on the network is excellent.

With a lightly loaded network, a slave node can comunicate with another slave

node at 2.4 milliseconds (per. complete transaction). The network utilization

flucuates according to slave node network demands.

Error recovery procedures are performed concurrently with all other

network activity. When one node goes down, the normal network activity

continues along with node recovery procedures for those nodes in error.

A IBM pc using the PC344 can be added to the system allowing high level

system control. This new master allows the programmer to concentrate on

the task at hand (cell control) instead of Bitbus network management.

Other computer systems such as the VAX can be encorporated into the system.

RAC commands can be sent from slave node to slave node. For those of you

who use the RAC commands, you know that you can only send one RAC command

at a time to a RAC task (because command/response is cleared, preventing

you from tracking individual command messages). Well that problem is also

fixed.

So what is the catch? Well there are two catches. The above system

requires and additional 2 bytes (using two data bytes) from the

message structure. Although this reduces the throughput per. message,

you must not forget the overall message throughput increase. The only

drawback is that your existing code must be protocol compatible with

the new master. The actual code change is minimal if the code was designed

correctly (i.e. shift data bytes down 2 bytes, don't hard code first

data byte location).

The other catch is that there is a limitation to the number of

communication paths between slaves. One task can communicate with

255 other tasks. Using only two tasks, one can communicate with

128 tasks while the other can communicate with 127 tasks, and so on.

There are 255 available "virtual connections" available to all tasks

which perform slave to slave communications. Local message passing is

not restricted.

That about covers it. If you would like more information about the

above information, or Bitbus hardware, please write or FAX:

Heath Smith

6919 Somerset Dr.

Brighton, MI

48116

FAX - 313-229-9072.

#########